Modbus RTU Temperature Monitoring & Relay Control Gateway

Embedded Systems Project — Technical Documentation

Prepared by: Zyad Hussien Mohamed Salah

Microcontroller Platform: ATmega32

Protocol: Modbus RTU over RS-485 (MAX485)

Toolchain: Atmel Studio

# 1. Project Overview

This project implements a Modbus RTU slave device on an ATmega32 microcontroller, communicating over an RS-485 physical layer via a MAX485 transceiver. The device continuously monitors ambient temperature through an onboard ADC channel, drives a relay-controlled load (e.g. a cooling fan or motor) based on a configurable temperature threshold, and exposes both the live temperature reading and the threshold value as Modbus registers accessible to a remote Master.

A 16x2 character LCD provides local, human-readable status (current temperature and motor/relay state) independently of the Modbus link, making the device usable as a standalone monitor as well as a networked slave node.

## 1.1 Key Objectives

- Implement the Modbus RTU frame format (Slave ID, Function Code, Data, CRC16) from scratch at the byte level, without any third-party Modbus stack.

- Support two operating modes: read-only Input Register mode and read/write Holding Register mode, selectable at compile time.

- Drive a half-duplex RS-485 bus correctly via the MAX485's Driver-Enable/Receiver-Enable control line, synchronized with UART transmission.

- Apply the same layered driver architecture (LIB / MCAL / HAL / APP) used throughout the author's embedded systems coursework, for consistency and reuse of existing DIO/UART/TIM0/ADC drivers.

## 1.2 Two Operating Modes

The project supports two mutually exclusive modes, selected via a compile-time macro in MODBUS_CFG.h:

| Mode | Macro | Behavior |
| --- | --- | --- |
| Input Register Mode | MODBUS_INPUT_REG_MODE | Temperature is read-only. The fixed threshold (35°C) is hardcoded and cannot be changed remotely. Function Code 0x04 (Read Input Registers) is used. |
| Holding Register Mode | MODBUS_HOLDING_REG_MODE | Temperature threshold becomes a writable register. A Master can both read (0x03) and write (0x06) the threshold value remotely, enabling dynamic reconfiguration of the trip point. |

# 2. System Architecture

## 2.1 Hardware Block Diagram

| Peripheral | Interface | Purpose |
| --- | --- | --- |
| MAX485 Transceiver | UART (PD0/PD1) + DIO control pin (PD2) | Converts UART TTL signals to/from the RS-485 differential bus; DE/RE pin toggled to switch between transmit and receive |
| Temperature Sensor (analog) | ADC Channel 2 | Provides the raw analog voltage proportional to ambient temperature |
| Relay Module | DIO (PC3) | Switches the controlled load (e.g. fan/motor) ON or OFF based on temperature comparison |
| 16x2 Character LCD | DIO | Local display of live temperature and motor/relay status |

## 2.2 RS-485 Half-Duplex Transmission Concept

Unlike RS-232 or a direct HC-05 UART link, RS-485 is a shared, half-duplex bus: only one device may drive the line at any instant. The MAX485_VoidSendByte() function handles this by asserting the Driver Enable pin HIGH immediately before transmitting a byte via UART, and returning it LOW immediately afterward — releasing the bus so the device (or another node) can receive.

MAX485_VoidSendByte(u8 copy_u8data)
{
    DE/RE pin -> HIGH   (enable transmitter, disable receiver)
    UART_VoidSendData(copy_u8data)
    DE/RE pin -> LOW    (disable transmitter, enable receiver)
}

This driver-enable toggling is the defining characteristic that separates an RS-485 HAL layer from a simple point-to-point UART driver, and is the reason MAX485 exists as its own HAL module built on top of the UART MCAL driver rather than being used directly.

# 3. Layered Software Architecture

The firmware follows the same four-layer architecture (LIB / MCAL / HAL / APP) used across the author's embedded systems projects, allowing the existing DIO, UART, TIM0, and ADC drivers to be reused without modification.

| Layer | Responsibility | Modules in This Project |
| --- | --- | --- |
| LIB | Shared types, bit-manipulation macros, interrupt vector definitions. | STD_TYPES.h, BIT_MATH.h, VECTOR_TABLE.h |
| MCAL | Register-level control of on-chip peripherals. | DIO, UART, TIM0, ADC |
| HAL | Drivers for external hardware, built on top of MCAL. | MAX485 (RS-485 transceiver control), Relay, LCD, LED, BTN |
| MODBUS | Protocol-specific engine: frame parsing, CRC16 validation, function-code dispatch. | MODBUS_voidInit(), MODBUS_voidTask() |
| APP | Top-level application logic tying everything together. | main.c — temperature polling, relay control loop, LCD updates |

Architecturally, the MODBUS module sits above HAL/MCAL and below APP: it depends on MAX485 (for the physical link) and Relay/ADC (to execute the requested actions), but exposes only two functions upward — MODBUS_voidInit() and MODBUS_voidTask() — keeping the protocol's internal framing and CRC logic fully encapsulated.

# 4. Modbus RTU Protocol Implementation

## 4.1 Frame Format

The implementation uses a fixed 8-byte request frame, matching the standard Modbus RTU structure for the single-coil and register-read function codes used in this project:

| Byte Offset | Field | Description |
| --- | --- | --- |
| 0 | Slave Address | Fixed at 0x01 for this device (MODBUS_SLAVE_ID) |
| 1 | Function Code | 0x03 (Read Holding Registers), 0x04 (Read Input Registers), or 0x05 (Write Single Coil) |
| 2–5 | Data | Register address / coil address and value fields, per standard Modbus RTU framing |
| 6–7 | CRC16 | Little-endian CRC16 checksum (low byte first) covering bytes 0–5 |

## 4.2 CRC16 Validation

Every incoming frame is validated with the standard Modbus CRC16 algorithm (polynomial 0xA001, initial value 0xFFFF) before any function code is processed. Frames that fail Slave ID matching or CRC verification are silently discarded, which is the correct and expected behavior for a Modbus slave sharing a multi-drop bus with other devices.

static u16 MODBUS_u16ClacCRC(u8 *pu8data, u8 u8length)
{
    u16 crc = 0xFFFF;
    for each byte:
        crc ^= byte
        for 8 bits:
            if (crc & 0x0001) crc = (crc >> 1) ^ 0xA001;
            else               crc = (crc >> 1);
    return crc;
}

## 4.3 Supported Function Codes

| Code | Name | Implemented Behavior |
| --- | --- | --- |
| 0x03 | Read Holding Registers | Returns the current temperature reading (or configurable threshold, depending on context) as a 2-byte value |
| 0x04 | Read Input Registers | Returns the live temperature reading, computed from the ADC channel, as a 2-byte value |
| 0x05 | Write Single Coil | Turns the relay ON (value 0xFF00) or OFF (value 0x0000); echoes the request frame back as confirmation, per the Modbus specification |

## 4.4 Frame Reception Strategy

Incoming bytes are accumulated non-blocking (UART_u8ReceiveNoBlock()) inside MODBUS_voidTask(), which is polled continuously from the main application loop. This design keeps the Modbus engine fully cooperative with the rest of the application (temperature polling, LCD updates) rather than blocking the CPU while waiting for bytes — an important distinction from the blocking UART_u8ReceiveData() pattern used elsewhere in the author's UART-based projects.

- The first received byte is checked against the expected Slave ID; mismatched frames are discarded immediately without buffering, minimizing wasted cycles on traffic addressed to other devices on the bus.

- Once 8 bytes have been accumulated, the frame is handed to MODBUS_VoidProcessFrame() for CRC validation and dispatch.

# 5. Temperature Sensing and Relay Control Logic

The application polls the ADC on channel 2 every main-loop iteration and converts the raw 10-bit reading to a temperature value in degrees Celsius using a fixed linear scaling factor:

Temp (°C) = (ADC_Value × 500) / 1024

This scaling assumes a linear analog sensor with a 5V reference producing 10mV per degree Celsius (consistent with common analog temperature sensor ICs such as the LM35), where a full-scale ADC reading of 1023 corresponds to approximately 500°C on the sensor's output curve.

The relay is switched based on a direct comparison against the threshold — fixed at 35°C in Input Register mode, or the runtime-adjustable Loc_TempThreshold variable in Holding Register mode. State transitions (motor RUNNING / STOPPED) are also reflected on the LCD, with a guard flag (Loc_MotorState) preventing redundant LCD writes on every loop iteration when the state hasn't changed.

# 6. Bill of Materials

| Component | Quantity | Role |
| --- | --- | --- |
| ATmega32 Microcontroller | 1 | Main processing unit |
| MAX485 RS-485 Transceiver | 1 | Physical layer for Modbus RTU bus |
| Relay Module | 1 | Switches the temperature-controlled load |
| Analog Temperature Sensor | 1 | Provides input to ADC channel 2 |
| 16x2 Character LCD (HD44780-compatible) | 1 | Local status display |
| RS-485 Bus Termination / Wiring | As needed | Connects the slave to the Modbus Master |

# 7. Testing Notes and Known Limitations

- Local UART key-based simulation ('r'/'w' key presses) is present in main.c to emulate remote Master read/write requests during development, prior to full RS-485 Master integration testing.

- The CRC bytes embedded in the simulated response frames within main.c (0x31, 0xCA) are static placeholders matched to the specific fixed payload used during testing; a general-purpose Master would compute CRC dynamically per response, as MODBUS_PRog.c already does for the Function Code 0x05 and 0x03/0x04 response paths.

- Only Slave ID 0x01 is currently supported; the architecture (constant-driven MODBUS_SLAVE_ID) allows this to be extended to a configurable or multi-slave setup with minimal change.

- The device has been exercised as a Modbus Slave only; Master-side polling was tested via simulated key presses rather than a full Modbus Master/PC-side tool (e.g. Modbus Poll) integration.

- Full Modbus Master/PC-side validation was later carried out using Modbus Poll as the Master application, with Virtual Serial Ports Emulator (VSPE) used to create a virtual COM port pair bridging Modbus Poll to the target's UART on a PC without a physical RS-485 adapter, confirming correct read/write behavior against the slave's Input and Holding Registers.

# 8. Future Work

- Integrate with a PC-based Modbus Master tool over a USB-to-RS485 adapter for full end-to-end protocol validation (Done — validated using Modbus Poll with VSPE for virtual COM port bridging; remaining work is repeating the test over a physical USB-to-RS485 adapter).

- Extend addressable register map to support multiple sensors/relays per slave, rather than a single fixed temperature/relay pair.

- Add Modbus exception responses (e.g. Illegal Function, Illegal Data Address) for malformed or unsupported requests, improving compliance with the full Modbus RTU specification.

- Support multi-slave addressing on a shared RS-485 bus to demonstrate the multi-drop capability the protocol is designed for.